# Construction Products labelling

To supply construction products for UK/EU, and that the product is covered by a harmonized standard (in our case BS EN 771), a declaration of performance must be given, and this declaration must include the following information: 1) product type reference, 2) systems of assessment and verification of the consistency of performance of the product, 3) intended use or uses for the product, and 4) declared performance based on the assessment according to the applicable harmonised standard.

An example of the product type reference would look something like this: "BRK-RED-500-ENG", where:

- BRK denotes that the product is a brick.

- RED refers to the color of the brick.

- 500 might indicate the size or a specific feature like compressive strength (e.g., 500 psi).

- ENG could denote the type, such as "engineering brick."

Once the declaration of performance has been drawn up, the manufacturer must affix a CE marking or the UKCA marking to the product. The CE marking remains valid in the UK until 30 June 2025. The marking indicates that the performance of the product has been assessed and that it remains constant. The marking enables a construction product to be placed legally on the market in the EU and in the UK.

Apart from the UKCA and CE markings, there are some other obligations for the manufacturers, importers and distributors.

Manufacturers must do the following:

- Prepare technical documentation and, on its basis, draw up a declaration of performance and affix a UKCA/CE marking to the product.

- Ensure that the product maintains its conformity with the declaration of performance. Where manufacturers consider that the product no longer conforms with the declaration of performance, they must immediately take the necessary corrective measures or withdraw or recall the product from the market.

Importers/ distributors must do the following:

- Ensure that the product bears the UKCA/CE marking and is accompanied by the required documentation (declaration of performance).

- Avoid placing the product on the market if they consider it not in conformity with the declaration of performance.

- Ensure that the product maintains its conformity with the declaration of performance and that its transport or storage does not jeopardise its performance. Where a product does not conform with the declaration of performance, importers must immediately take the necessary corrective measures or withdraw or recall the product from the market.

The above information are from the following regulations:

UK –	 Construction Products (Amendment etc.) (EU Exit) Regulations 2019

Construction Products (Amendment etc.) (EU Exit) Regulations 2020

EU – 	Regulation (EU) No 305/2011 – harmonised conditions for the marketing of construction products